JRM + ESP32

- Complete System Flow

POWER ON

↓

Language Selection

↓

Route Selection

↓

GPS Fast Lock

↓

Coordinate Matching

↓

Zone Detection

↓

LED Control

↓

Voice Announcement

↓

Overspeed Alert

↓

Continuous Monitoring

GPS Based Intelligent Audio Alert System

C

- What Happens at Each Stage

- When 5V power is supplied

- ESP32 boots

- GPS module starts

- DFPlayer initializes

- LEDs initialize

- Route database loads from memory

- Language Selection

1 = Marathi , 2 = Hindi , 3 = English  . Based on selected language:

Language Folder Selected.

- Route selection

Route 510 , Route 511 ,Route 512 .from Loaded information contains: Coordinates ,Zone IDs , Speed limits.

- GPS Lock

- Latitude Available ,Longitude Available ,Satellite Lock Available .

- Coordinate Matching

- Current co-ordinates matches with load route files co-ordinate .

- Zone Detection and LED Control

Based on detected zone  LED indication changes to Red ,Yellow, Green .

- Voice Announcement

- As per zone zone changing  voice command  and overspeed  command .

- Overspeed Alert

Every zone may have a predefined speed limit as vehical speed exceed that limit overspeed buzzer start beeping till speed becoms normal again .

- Components Used

- Hardware  :

- ESP32 Development Board

- GPS Module (L80)

- DFPlayer Mini MP3 Module

- MicroSD Card (8 GB)

- Speaker (4Ω 3W)

- 5 V DC Power Supply

- LED’s (Red,Yellow,Green)

- Current Limiting Resistors

- Buzzer (5V)

- Connecting Wires and Connectors

Software :

- Arduino IDE

- ESP32 Board Package

- TinyGPS++ Library

- DFRo

- LittleFS Library

- UART Communication Libraries

- Main Features Implemented

✅GPS Fast Lock

✅ Coordinate Matching

✅ Zone Detection

✅ LED Indication

✅ Voice Announcements

✅ Overspeed Detection

✅ Speed Limit Monitoring

✅ Continuous GPS Tracking

✅ Multi-route Support

✅ Multi-language Support

- Connections

- GPS Module → ESP32

GPS VCC  → ESP32 5V
GPS GND  → ESP32 GND
GPS TX   → ESP32 GPIO 16(RX1)
GPS RX   → ESP32 GPIO 17(TX1)

- SD Card Module → ESP32

VCC  → 3.3V

GND →  GND

MISO  → GPIO19

MOSI  → GPIO23

SCK  →  GPIO18

CS  → GPIO5

- MAX98357 → ESP32

VIN  → 5V

GND → GND
  BCLK → GPIO 32

LRC  →  GPIO 33

DIN→ GPIO 22

- Speaker → MAX98357

Speaker + → +ve
Speaker - → -ve

- Buzzer → ESP32

VCC → 5 V

GND → GPIO 14

- LED’s → ESP32

Red (+ve) → GPIO 25

Yellow (+ve) → GPIO 26

Green (+ve) → GPIO 27

LED’s (-ve) →  ESP32 GND

- MicroSD Card

Insert MicroSD Card into DFPlayer Mini  & Store voice files inside card such as:

//English                //Hindi                //Marathi                    Zone  Sound

0001.mp3                0101.mp3          0201.mp3                      For Green 
0002.mp3                0102.mp3          0202.mp3                      For Yellow 
0003.mp3                0103.mp3          0203.mp3                      For Red

0004.mp3                0104.mp3          0204.mp3                      Over speed

- Power Connections

ESP32

5V Supply → ESP32 VIN
GND        → ESP32 GND

GPS

ESP32 3.3V → GPS VCC
ESP32 GND  → GPS GND

DFPlayer

5V Supply → DFPlayer VCC
GND        → DFPlayer GND

- JRM Panel

-ve

BUZZER

R1,R2,R3(220Ω)

+ve

YELLOW

GREEN

RED

/*

* ============================================================

*   GPS BASED INTELLIGENT AUDIO ALERT SYSTEM

*   For School / College Bus Safety

* ============================================================

*   Hardware : ESP32 DevKit V1

*   GPS      : L80 GPS Module

*   Audio    : MAX98357 I2S Amplifier

*   Storage  : Micro SD Card Module

*   BT       : Bluetooth Classic Serial

*   Author   : Production Build v1.0

*   Date     : 2026

* ============================================================

*

*  PIN CONFIGURATION:

*   GPS RX        -> GPIO17

*   GPS TX        -> GPIO16

*   Push Button   -> GPIO13

*   Green LED     -> GPIO27

*   Yellow LED    -> GPIO26

*   Red LED       -> GPIO25

*   Buzzer        -> GPIO14

*   MAX98357 DIN  -> GPIO22

*   MAX98357 BCLK -> GPIO32

*   MAX98357 LRC  -> GPIO33

*   SD CS         -> GPIO5

*   SD MOSI       -> GPIO23

*   SD MISO       -> GPIO19

*   SD SCK        -> GPIO18

* ============================================================

*/

// ─────────────────────────────────────────────

//  INCLUDES

// ─────────────────────────────────────────────

#include <Arduino.h>

#include <TinyGPSPlus.h>

#include <HardwareSerial.h>

#include <BluetoothSerial.h>

#include <SD.h>

#include <SPI.h>

#include <Audio.h>          // ESP32-audioI2S by Schreibfaul1

#include <math.h>

// ─────────────────────────────────────────────

//  PIN DEFINITIONS

// ─────────────────────────────────────────────

#define PIN_GPS_RX       17

#define PIN_GPS_TX       16

#define PIN_BUTTON       13

#define PIN_LED_GREEN    27

#define PIN_LED_YELLOW   26

#define PIN_LED_RED      25

#define PIN_BUZZER       14

#define PIN_I2S_DIN      22

#define PIN_I2S_BCLK     32

#define PIN_I2S_LRC      33

#define PIN_SD_CS         5

#define PIN_SD_MOSI      23

#define PIN_SD_MISO      19

#define PIN_SD_SCK       18

// ─────────────────────────────────────────────

//  PROJECT CONSTANTS

// ─────────────────────────────────────────────

#define BLUETOOTH_NAME        "JRM+ESP32"

#define MATCH_RADIUS_M        100.0          // metres

#define OVERSPEED_COOLDOWN_MS 10000UL        // 10 seconds

#define BUTTON_DEBOUNCE_MS    50UL

#define BUTTON_TIMEOUT_MS     3000UL         // wait 3 s after last press

#define GPS_BAUD              9600

#define BUZZER_BEEP_ON_MS     150UL

#define BUZZER_BEEP_OFF_MS    150UL

#define BUZZER_BEEP_COUNT     3

#define EARTH_RADIUS_M        6371000.0

// Speed limits per zone (km/h)

#define SPEED_LIMIT_GREEN     65.0

#define SPEED_LIMIT_YELLOW    40.0

#define SPEED_LIMIT_RED       30.0

// ─────────────────────────────────────────────

//  ENUMERATIONS

// ─────────────────────────────────────────────

enum Language {

LANG_NONE    = 0,

LANG_ENGLISH = 1,

LANG_HINDI   = 2,

LANG_MARATHI = 3

};

enum Zone {

ZONE_NONE   = 0,

ZONE_GREEN  = 'G',

ZONE_YELLOW = 'Y',

ZONE_RED    = 'R'

};

enum SystemState {

STATE_LANGUAGE_SELECT,

STATE_BT_ROUTE_SELECT,

STATE_LOAD_ROUTE,

STATE_GPS_LOCK,

STATE_MONITORING

};

// ─────────────────────────────────────────────

//  DATA STRUCTURES

// ─────────────────────────────────────────────

struct RoutePoint {

double latitude;

double longitude;

char   zone;       // 'G', 'Y', 'R'

};

// ─────────────────────────────────────────────

//  GLOBALS – Hardware Instances

// ─────────────────────────────────────────────

TinyGPSPlus      gps;

HardwareSerial   gpsSerial(2);   // UART2 for GPS

BluetoothSerial  btSerial;

Audio            audio;

// ─────────────────────────────────────────────

//  GLOBALS – System State

// ─────────────────────────────────────────────

SystemState      systemState       = STATE_LANGUAGE_SELECT;

Language         selectedLanguage  = LANG_NONE;

String           selectedRoute     = "";

String           routeFilePath     = "";

// Route points loaded from SD card

static const uint16_t MAX_ROUTE_POINTS = 500;

RoutePoint       routePoints[MAX_ROUTE_POINTS];

uint16_t         routePointCount   = 0;

// GPS data

double           currentLat        = 0.0;

double           currentLon        = 0.0;

double           currentSpeed      = 0.0;   // km/h

// Zone tracking

char             currentZone       = ZONE_NONE;

char             lastAnnouncedZone = ZONE_NONE;

// Overspeed tracking

bool             overspeedActive   = false;

unsigned long    overspeedCooldownStart = 0;

// Button tracking

int              buttonPressCount  = 0;

unsigned long    lastButtonPress   = 0;

bool             buttonWasHigh     = true;    // INPUT_PULLUP – idle HIGH

unsigned long    lastDebounce      = 0;

bool             languageConfirmed = false;

// Buzzer non-blocking state

bool             buzzerActive      = false;

int              buzzerBeepsLeft   = 0;

bool             buzzerToneOn      = false;

unsigned long    buzzerLastChange  = 0;

// Audio state

bool             audioPlaying      = false;

String           lastAudioFile     = "";

// Bluetooth input buffer

String           btBuffer          = "";

// Millis timestamps

unsigned long    lastGPSRead       = 0;

unsigned long    lastZoneCheck     = 0;

// ─────────────────────────────────────────────

//  FORWARD DECLARATIONS

// ─────────────────────────────────────────────

void     selectLanguage();

void     readBluetoothRoute();

bool     loadRouteFile(const String& path);

void     readGPS();

double   calculateDistance(double lat1, double lon1, double lat2, double lon2);

char     detectZone();

void     updateLEDs(char zone);

void     playAudio(const String& filename);

void     checkOverspeed(char zone, double speed);

void     beepBuzzer();

void     serviceBuzzer();

String   getZoneAudioFile(char zone);

String   getOverspeedAudioFile();

double   getSpeedLimit(char zone);

void     printDebugInfo();

void     blinkLED(int pin, int times, int onMs, int offMs);

// ─────────────────────────────────────────────

//  SETUP

// ─────────────────────────────────────────────

void setup() {

Serial.begin(115200);

Serial.println(F("========================================"));

Serial.println(F("  GPS Bus Safety System – Starting Up  "));

Serial.println(F("========================================"));

// GPIO init

pinMode(PIN_BUTTON,     INPUT_PULLUP);

pinMode(PIN_LED_GREEN,  OUTPUT);

pinMode(PIN_LED_YELLOW, OUTPUT);

pinMode(PIN_LED_RED,    OUTPUT);

pinMode(PIN_BUZZER,     OUTPUT);

digitalWrite(PIN_LED_GREEN,  LOW);

digitalWrite(PIN_LED_YELLOW, LOW);

digitalWrite(PIN_LED_RED,    LOW);

digitalWrite(PIN_BUZZER,     LOW);

// GPS Serial

gpsSerial.begin(GPS_BAUD, SERIAL_8N1, PIN_GPS_RX, PIN_GPS_TX);

Serial.println(F("[GPS] Serial started on UART2"));

// Bluetooth

btSerial.begin(BLUETOOTH_NAME);

Serial.print(F("[BT] Started as: "));

Serial.println(BLUETOOTH_NAME);

// SD Card

SPI.begin(PIN_SD_SCK, PIN_SD_MISO, PIN_SD_MOSI, PIN_SD_CS);

if (!SD.begin(PIN_SD_CS)) {

Serial.println(F("[SD] FAILED – check wiring/card!"));

for (int i = 0; i < 5; i++) {

digitalWrite(PIN_LED_RED, HIGH); delay(200);

digitalWrite(PIN_LED_RED, LOW);  delay(200);

}

} else {

Serial.println(F("[SD] Card mounted OK"));

}

// I2S Audio

audio.setPinout(PIN_I2S_BCLK, PIN_I2S_LRC, PIN_I2S_DIN);

audio.setVolume(18);   // 0–21

Serial.println(F("[AUDIO] I2S configured"));

// Startup visual confirmation – all LEDs ON briefly

digitalWrite(PIN_LED_GREEN,  HIGH);

digitalWrite(PIN_LED_YELLOW, HIGH);

digitalWrite(PIN_LED_RED,    HIGH);

delay(600);

digitalWrite(PIN_LED_GREEN,  LOW);

digitalWrite(PIN_LED_YELLOW, LOW);

digitalWrite(PIN_LED_RED,    LOW);

Serial.println(F("[SYS] Press button: 1=English | 2=Hindi | 3=Marathi"));

Serial.println(F("[SYS] Waiting for language selection..."));

}

// ─────────────────────────────────────────────

//  MAIN LOOP

// ─────────────────────────────────────────────

void loop() {

// Always service audio loop (required by ESP32-audioI2S)

audio.loop();

// Always service non-blocking buzzer

serviceBuzzer();

// State machine

switch (systemState) {

// ── 1. LANGUAGE SELECTION ──────────────────

case STATE_LANGUAGE_SELECT:

selectLanguage();

break;

// ── 2. BLUETOOTH ROUTE SELECTION ──────────

case STATE_BT_ROUTE_SELECT:

readBluetoothRoute();

break;

// ── 3. LOAD ROUTE FILE ─────────────────────

case STATE_LOAD_ROUTE:

if (loadRouteFile(routeFilePath)) {

Serial.printf("[ROUTE] Loaded %d points from %s\n",

routePointCount, routeFilePath.c_str());

systemState = STATE_GPS_LOCK;

Serial.println(F("[GPS] Waiting for GPS lock..."));

} else {

Serial.println(F("[ERROR] Route load failed – check SD card!"));

blinkLED(PIN_LED_YELLOW, 4, 200, 200);

systemState = STATE_BT_ROUTE_SELECT;

}

break;

// ── 4. WAIT FOR GPS FIX ───────────────────

case STATE_GPS_LOCK:

readGPS();

if (gps.location.isValid() && gps.location.age() < 2000) {

currentLat   = gps.location.lat();

currentLon   = gps.location.lng();

currentSpeed = gps.speed.kmph();

Serial.printf("[GPS] Lock acquired: %.6f, %.6f  Speed: %.1f km/h\n",

currentLat, currentLon, currentSpeed);

blinkLED(PIN_LED_GREEN, 3, 150, 150);

systemState = STATE_MONITORING;

Serial.println(F("[SYS] === MONITORING STARTED ==="));

}

break;

// ── 5. CONTINUOUS MONITORING ──────────────

case STATE_MONITORING:

readGPS();

if (millis() - lastZoneCheck >= 500) {  // check every 500ms

lastZoneCheck = millis();

char newZone = detectZone();

if (newZone != ZONE_NONE && newZone != currentZone) {

currentZone = newZone;

updateLEDs(currentZone);

// Play zone audio only when zone changes

if (currentZone != lastAnnouncedZone) {

lastAnnouncedZone = currentZone;

String audioFile = getZoneAudioFile(currentZone);

playAudio(audioFile);

}

}

checkOverspeed(currentZone, currentSpeed);

printDebugInfo();

}

break;

}

}

// ─────────────────────────────────────────────

//  FUNCTION: selectLanguage()

//  - Reads button presses with debounce

//  - 3-second timeout after last press

//  - 1=English, 2=Hindi, 3=Marathi

// ─────────────────────────────────────────────

void selectLanguage() {

bool buttonNow = (digitalRead(PIN_BUTTON) == LOW);  // LOW = pressed (pull-up)

// Debounce

if (buttonNow != !buttonWasHigh) {

lastDebounce = millis();

}

if ((millis() - lastDebounce) > BUTTON_DEBOUNCE_MS) {

if (buttonNow && buttonWasHigh) {

// Falling edge – button pressed

buttonWasHigh    = false;

lastButtonPress  = millis();

buttonPressCount++;

// Visual feedback per press

digitalWrite(PIN_LED_GREEN, HIGH);

delay(80);

digitalWrite(PIN_LED_GREEN, LOW);

Serial.printf("[BTN] Press #%d detected\n", buttonPressCount);

} else if (!buttonNow && !buttonWasHigh) {

buttonWasHigh = true;  // button released

}

}

// After 3-second timeout, confirm selection

if (buttonPressCount > 0 &&

(millis() - lastButtonPress) >= BUTTON_TIMEOUT_MS) {

switch (buttonPressCount) {

case 1:

selectedLanguage = LANG_ENGLISH;

Serial.println(F("[LANG] Selected: ENGLISH"));

break;

case 2:

selectedLanguage = LANG_HINDI;

Serial.println(F("[LANG] Selected: HINDI"));

break;

case 3:

selectedLanguage = LANG_MARATHI;

Serial.println(F("[LANG] Selected: MARATHI"));

break;

default:

Serial.printf("[LANG] Invalid count %d – defaulting to English\n",

buttonPressCount);

selectedLanguage = LANG_ENGLISH;

break;

}

// Confirm with LED flash

int pin = (selectedLanguage == LANG_ENGLISH) ? PIN_LED_GREEN :

(selectedLanguage == LANG_HINDI)   ? PIN_LED_YELLOW :

PIN_LED_RED;

blinkLED(pin, 3, 200, 100);

buttonPressCount = 0;

systemState      = STATE_BT_ROUTE_SELECT;

Serial.println(F("[SYS] Language confirmed. Waiting for BT route..."));

Serial.println(F("[BT] Send: R500, R510, or R520"));

}

}

// ─────────────────────────────────────────────

//  FUNCTION: readBluetoothRoute()

//  - Listens for "R500","R510","R520" over BT

//  - Maps route code to SD file path

// ─────────────────────────────────────────────

void readBluetoothRoute() {

while (btSerial.available()) {

char c = (char)btSerial.read();

if (c == '\n' || c == '\r') {

btBuffer.trim();

btBuffer.toUpperCase();

Serial.print(F("[BT] Received: "));

Serial.println(btBuffer);

if (btBuffer == "R500") {

selectedRoute  = "R500";

routeFilePath  = "/Route500.txt";

} else if (btBuffer == "R510") {

selectedRoute  = "R510";

routeFilePath  = "/Route510.txt";

} else if (btBuffer == "R520") {

selectedRoute  = "R520";

routeFilePath  = "/Route520.txt";

} else {

btSerial.println("ERROR: Send R500, R510, or R520");

Serial.println(F("[BT] Unknown route command"));

btBuffer = "";

return;

}

btSerial.print("Route selected: ");

btSerial.println(selectedRoute);

Serial.printf("[ROUTE] Selected %s -> %s\n",

selectedRoute.c_str(), routeFilePath.c_str());

btBuffer    = "";

systemState = STATE_LOAD_ROUTE;

return;

} else {

btBuffer += c;

if (btBuffer.length() > 20) btBuffer = "";  // overflow protection

}

}

}

// ─────────────────────────────────────────────

//  FUNCTION: loadRouteFile()

//  - Opens CSV file from SD card

//  - Parses: latitude,longitude,zone

//  - Stores into routePoints[]

// ─────────────────────────────────────────────

bool loadRouteFile(const String& path) {

Serial.printf("[SD] Opening: %s\n", path.c_str());

File f = SD.open(path.c_str());

if (!f) {

Serial.printf("[SD] Cannot open file: %s\n", path.c_str());

return false;

}

routePointCount = 0;

String line;

while (f.available() && routePointCount < MAX_ROUTE_POINTS) {

line = f.readStringUntil('\n');

line.trim();

// Skip empty lines and comments

if (line.length() == 0 || line.startsWith("#")) continue;

// Parse: lat,lon,zone

int comma1 = line.indexOf(',');

int comma2 = line.indexOf(',', comma1 + 1);

if (comma1 < 0 || comma2 < 0) {

Serial.printf("[SD] Malformed line: %s\n", line.c_str());

continue;

}

double lat  = line.substring(0, comma1).toDouble();

double lon  = line.substring(comma1 + 1, comma2).toDouble();

char   zone = line.charAt(comma2 + 1);

// Validate zone character

if (zone != 'G' && zone != 'Y' && zone != 'R') {

Serial.printf("[SD] Unknown zone '%c' – skipping\n", zone);

continue;

}

routePoints[routePointCount].latitude  = lat;

routePoints[routePointCount].longitude = lon;

routePoints[routePointCount].zone      = zone;

routePointCount++;

}

f.close();

if (routePointCount == 0) {

Serial.println(F("[SD] No valid route points found!"));

return false;

}

Serial.printf("[SD] Loaded %d route points.\n", routePointCount);

return true;

}

// ─────────────────────────────────────────────

//  FUNCTION: readGPS()

//  - Feeds GPS serial bytes to TinyGPSPlus

//  - Updates currentLat, currentLon, currentSpeed

// ─────────────────────────────────────────────

void readGPS() {

while (gpsSerial.available() > 0) {

char c = gpsSerial.read();

gps.encode(c);

}

if (gps.location.isUpdated()) {

currentLat   = gps.location.lat();

currentLon   = gps.location.lng();

currentSpeed = gps.speed.isValid() ? gps.speed.kmph() : 0.0;

}

}

// ─────────────────────────────────────────────

//  FUNCTION: calculateDistance()

//  - Haversine formula

//  - Returns distance between two GPS coords (metres)

// ─────────────────────────────────────────────

double calculateDistance(double lat1, double lon1,

double lat2, double lon2) {

double dLat = radians(lat2 - lat1);

double dLon = radians(lon2 - lon1);

double a = sin(dLat / 2.0) * sin(dLat / 2.0) +

cos(radians(lat1)) * cos(radians(lat2)) *

sin(dLon / 2.0) * sin(dLon / 2.0);

double c = 2.0 * atan2(sqrt(a), sqrt(1.0 - a));

return EARTH_RADIUS_M * c;

}

// ─────────────────────────────────────────────

//  FUNCTION: detectZone()

//  - Compares current GPS position to all route points

//  - Returns zone of nearest match within MATCH_RADIUS_M

//  - Returns ZONE_NONE if no point within radius

// ─────────────────────────────────────────────

char detectZone() {

if (routePointCount == 0)    return ZONE_NONE;

if (!gps.location.isValid()) return ZONE_NONE;

double minDist     = 1e9;

char   matchedZone = ZONE_NONE;

for (uint16_t i = 0; i < routePointCount; i++) {

double dist = calculateDistance(currentLat, currentLon,

routePoints[i].latitude,

routePoints[i].longitude);

if (dist <= MATCH_RADIUS_M && dist < minDist) {

minDist     = dist;

matchedZone = routePoints[i].zone;

}

}

return matchedZone;

}

// ─────────────────────────────────────────────

//  FUNCTION: updateLEDs()

//  - Turns ON correct LED, turns off others

// ─────────────────────────────────────────────

void updateLEDs(char zone) {

digitalWrite(PIN_LED_GREEN,  LOW);

digitalWrite(PIN_LED_YELLOW, LOW);

digitalWrite(PIN_LED_RED,    LOW);

switch (zone) {

case ZONE_GREEN:

digitalWrite(PIN_LED_GREEN, HIGH);

Serial.println(F("[LED] GREEN ON"));

break;

case ZONE_YELLOW:

digitalWrite(PIN_LED_YELLOW, HIGH);

Serial.println(F("[LED] YELLOW ON"));

break;

case ZONE_RED:

digitalWrite(PIN_LED_RED, HIGH);

Serial.println(F("[LED] RED ON"));

break;

default:

Serial.println(F("[LED] ALL OFF (no zone)"));

break;

}

}

// ─────────────────────────────────────────────

//  FUNCTION: playAudio()

//  - Plays MP3 file from SD card via I2S / MAX98357

//  - Prevents re-triggering the same file

// ─────────────────────────────────────────────

void playAudio(const String& filename) {

if (filename == lastAudioFile && audioPlaying) {

return;  // already playing this file

}

String fullPath = "/" + filename;

Serial.printf("[AUDIO] Playing: %s\n", fullPath.c_str());

audio.connecttoFS(SD, fullPath.c_str());

lastAudioFile = filename;

audioPlaying  = true;

}

// ESP32-audioI2S library callback – fires when MP3 ends

void audio_eof_mp3(const char* info) {

audioPlaying  = false;

lastAudioFile = "";

Serial.printf("[AUDIO] Finished: %s\n", info);

}

void audio_info(const char* info) {

// Uncomment below for verbose audio debug:

// Serial.printf("[AUDIO_INFO] %s\n", info);

}

// ─────────────────────────────────────────────

//  FUNCTION: checkOverspeed()

//  - Compares speed against zone's speed limit

//  - Triggers buzzer × 3 + voice alert

//  - 10-second cooldown between alerts

// ─────────────────────────────────────────────

void checkOverspeed(char zone, double speed) {

if (zone == ZONE_NONE) return;

double limit    = getSpeedLimit(zone);

bool   speeding = (speed > limit);

if (speeding) {

unsigned long now      = millis();

bool          cooldownOk = (now - overspeedCooldownStart) >= OVERSPEED_COOLDOWN_MS;

if (!overspeedActive || cooldownOk) {

overspeedActive        = true;

overspeedCooldownStart = now;

Serial.printf("[OVERSPEED] Speed %.1f > limit %.1f km/h\n", speed, limit);

beepBuzzer();

String overspeedFile = getOverspeedAudioFile();

playAudio(overspeedFile);

}

} else {

overspeedActive = false;

}

}

// ─────────────────────────────────────────────

//  FUNCTION: beepBuzzer()

//  - Starts non-blocking 3-beep sequence

// ─────────────────────────────────────────────

void beepBuzzer() {

if (buzzerActive) return;  // already running

buzzerActive     = true;

buzzerBeepsLeft  = BUZZER_BEEP_COUNT;

buzzerToneOn     = false;

buzzerLastChange = millis();

Serial.println(F("[BUZZER] 3-beep started"));

}

// ─────────────────────────────────────────────

//  FUNCTION: serviceBuzzer()

//  - Called every loop() – no delay() used

//  - Handles buzzer ON/OFF timing with millis()

// ─────────────────────────────────────────────

void serviceBuzzer() {

if (!buzzerActive) return;

unsigned long now     = millis();

unsigned long elapsed = now - buzzerLastChange;

if (!buzzerToneOn) {

if (elapsed >= BUZZER_BEEP_OFF_MS) {

digitalWrite(PIN_BUZZER, HIGH);

buzzerToneOn     = true;

buzzerLastChange = now;

}

} else {

if (elapsed >= BUZZER_BEEP_ON_MS) {

digitalWrite(PIN_BUZZER, LOW);

buzzerToneOn     = false;

buzzerLastChange = now;

buzzerBeepsLeft--;

if (buzzerBeepsLeft <= 0) {

buzzerActive = false;

Serial.println(F("[BUZZER] Done"));

}

}

}

}

// ─────────────────────────────────────────────

//  HELPER: getZoneAudioFile()

//  - Returns correct MP3 filename based on

//    current zone AND selected language

// ─────────────────────────────────────────────

String getZoneAudioFile(char zone) {

// English : 0001–0004

// Hindi   : 0101–0104

// Marathi : 0201–0204

uint16_t base = 0;

switch (selectedLanguage) {

case LANG_ENGLISH: base = 0;   break;

case LANG_HINDI:   base = 100; break;

case LANG_MARATHI: base = 200; break;

default:           base = 0;   break;

}

uint16_t num = 0;

switch (zone) {

case ZONE_GREEN:  num = base + 1; break;

case ZONE_YELLOW: num = base + 2; break;

case ZONE_RED:    num = base + 3; break;

default:          return "";

}

char buf[12];

snprintf(buf, sizeof(buf), "%04d.mp3", num);

return String(buf);

}

// ─────────────────────────────────────────────

//  HELPER: getOverspeedAudioFile()

//  - Returns overspeed MP3 filename for language

// ─────────────────────────────────────────────

String getOverspeedAudioFile() {

uint16_t base = 0;

switch (selectedLanguage) {

case LANG_ENGLISH: base = 0;   break;

case LANG_HINDI:   base = 100; break;

case LANG_MARATHI: base = 200; break;

default:           base = 0;   break;

}

char buf[12];

snprintf(buf, sizeof(buf), "%04d.mp3", base + 4);

return String(buf);

}

// ─────────────────────────────────────────────

//  HELPER: getSpeedLimit()

//  - Returns km/h speed limit for given zone

// ─────────────────────────────────────────────

double getSpeedLimit(char zone) {

switch (zone) {

case ZONE_GREEN:  return SPEED_LIMIT_GREEN;

case ZONE_YELLOW: return SPEED_LIMIT_YELLOW;

case ZONE_RED:    return SPEED_LIMIT_RED;

default:          return 999.0;

}

}

// ─────────────────────────────────────────────

//  FUNCTION: printDebugInfo()

//  - Prints full status to Serial Monitor every 2s

// ─────────────────────────────────────────────

void printDebugInfo() {

static unsigned long lastPrint = 0;

if (millis() - lastPrint < 2000) return;

lastPrint = millis();

Serial.println(F("─────────────────────────────────────────"));

Serial.printf("[DEBUG] Language    : %s\n",

selectedLanguage == LANG_ENGLISH ? "English" :

selectedLanguage == LANG_HINDI   ? "Hindi"   : "Marathi");

Serial.printf("[DEBUG] Route       : %s\n", selectedRoute.c_str());

Serial.printf("[DEBUG] Latitude    : %.6f\n", currentLat);

Serial.printf("[DEBUG] Longitude   : %.6f\n", currentLon);

Serial.printf("[DEBUG] Speed       : %.1f km/h\n", currentSpeed);

Serial.printf("[DEBUG] Zone        : %c\n",

(currentZone == ZONE_NONE) ? '-' : currentZone);

Serial.printf("[DEBUG] Speed Limit : %.0f km/h\n", getSpeedLimit(currentZone));

Serial.printf("[DEBUG] Audio File  : %s\n", lastAudioFile.c_str());

Serial.printf("[DEBUG] Overspeed   : %s\n", overspeedActive ? "YES" : "NO");

Serial.printf("[DEBUG] GPS Sats    : %d\n",

gps.satellites.isValid() ? (int)gps.satellites.value() : 0);

Serial.println(F("─────────────────────────────────────────"));

}

// ─────────────────────────────────────────────

//  UTILITY: blinkLED()

//  - Simple blocking blink (only during state transitions)

// ─────────────────────────────────────────────

void blinkLED(int pin, int times, int onMs, int offMs) {

for (int i = 0; i < times; i++) {

digitalWrite(pin, HIGH);

delay(onMs);

digitalWrite(pin, LOW);

if (i < times - 1) delay(offMs);

}

}